A Comprehensive Guide to Building an Intelligent Traffic Signal Controller

Part I: Foundations of Traffic Dynamics and Control

Chapter 1: The Anatomy of Traffic Congestion

To effectively control traffic, one must first understand its fundamental principles. Traffic is not merely a collection of cars; it is a complex system governed by mathematical relationships. This chapter deconstructs the science of traffic flow, providing the theoretical bedrock upon which an intelligent control system is built.

1.1 The Language of Traffic: Core Elements

Traffic engineering uses a precise vocabulary to describe the state of a roadway. The primary elements of any traffic stream are flow, density, and speed. Understanding these variables and their interplay is the first step toward modeling and managing traffic.

- Flow (q): Often used interchangeably with volume, flow represents the number of vehicles passing a specific point on a roadway over a unit of time. It is typically expressed in vehicles per hour (veh/h). While volume generally refers to a count over a full hour, flow rate can be calculated for shorter intervals (e.g., 15 minutes) and extrapolated to an hourly rate, providing a more dynamic measure of traffic conditions. The formula to determine the equivalent hourly flow from a shorter count is:

q=n×T3600​

where n is the number of vehicles counted in T seconds.

- Density (k): Also known as concentration, density is the number of vehicles occupying a given length of roadway at a specific instant in time. It is usually measured in vehicles per kilometer (veh/km) or vehicles per mile. Density provides a snapshot of how crowded a road segment is.

- Speed (v): Speed is the rate of motion of vehicles, but for traffic analysis, it is crucial to distinguish between two types of mean speeds :

  - Time Mean Speed (vt​): This is the arithmetic average of the speeds of all vehicles passing a single point on a highway over a period. It is what a stationary radar gun would measure.

  - Space Mean Speed (vs​): This is the average speed of vehicles over a certain length of roadway at a given time. It is calculated as the harmonic mean of vehicle speeds and is always less than or equal to the time mean speed. Space mean speed is the more relevant metric for traffic flow analysis because it accurately reflects the overall state of a road segment and is the speed used in the fundamental traffic flow equation. It can be calculated as:

vs​=∑i=1n​ti​nL​

where n is the number of vehicles, L is the length of the highway section, and ti​ is the time it takes the i-th vehicle to travel that section.

These macroscopic properties are emergent from microscopic behaviors, which are described by headway. Time headway (ht​) is the time interval between the front bumpers of two successive vehicles as they pass a point, while space headway (ds​) is the distance between them. In essence, flow is the inverse of the average time headway, and density is the inverse of the average space headway.

1.2 The Fundamental Diagram of Traffic Flow

The relationships between flow, speed, and density are not independent; they are intrinsically linked, as illustrated by the Fundamental Diagram of Traffic Flow. This diagram is a set of three graphs that are central to understanding the different states of traffic. The core relationship that binds these elements is:

q=k×vs​

This equation states that flow is the product of density and space mean speed.

- Speed-Density Relationship: This relationship is generally modeled as linear with a negative slope. When density is near zero, vehicles can travel at their maximum desired speed, known as the

free-flow speed (vf​). As density increases, vehicles are forced to slow down. The relationship continues until density reaches its maximum, the jam density (kj​), at which point all vehicles are stopped (bumper-to-bumper), and speed is zero.

- Flow-Density Relationship: This diagram is arguably the most important for diagnosing the state of traffic. It is typically represented by a parabolic or triangular curve.

  - At zero density, flow is zero because there are no vehicles.

  - As density increases from zero, flow also increases. More cars on the road lead to more cars passing a point per hour.

  - This continues until a critical point is reached. The maximum flow rate a roadway can handle is called its capacity (qmax​). This occurs at an optimum density (ko​) and an optimum speed (vo​).

  - If density increases beyond this optimum point, interactions between vehicles cause speeds to drop faster than density increases, leading to a decrease in flow. This is the "congested" branch of the curve.

  - Finally, when density reaches the jam density (kj​), vehicles are at a standstill, and flow once again becomes zero. This paradox—that putting more cars on the road can lead to fewer cars getting through—is the essence of traffic congestion.

- Speed-Flow Relationship: This diagram shows that for any given flow rate below the roadway's capacity, two different speeds are possible. One speed corresponds to a low-density, free-flow condition, while the other corresponds to a high-density, congested condition. The peak of the curve represents the capacity, which occurs at a single, optimal speed.

1.3 The Birth of a Queue: From Theory to Reality

The theoretical models of traffic flow directly explain the real-world phenomenon of queuing at intersections. Congestion begins when the traffic state transitions from stable (free-flow) to unstable (congested), which happens when the density of arriving vehicles surpasses the roadway's capacity to discharge them.

An intersection acts as a service point in a queuing system. Queues form when the arrival rate of vehicles (λ) is greater than the service rate (μ). For a signalized intersection, the service rate is the number of vehicles that can pass through during a green light. This rate is not constant; it is directly controlled by the traffic signal's timing. When an approach has a red light, its service rate is zero, and a queue inevitably builds. When it turns green, the service rate becomes positive (determined by the saturation flow rate), and the queue begins to discharge.

A "cycle failure" occurs when the queue that built up during the red phase is not fully cleared by the end of the subsequent green phase. This residual or "overflow" queue is then carried over to the next cycle, leading to progressively longer queues and delays. The entire purpose of an intelligent traffic controller is to dynamically manage the service rate (

μ) for all competing approaches by allocating green time effectively, thereby keeping the traffic intensity (ρ=λ/μ) below the critical value of 1 for as long as possible and preventing such cycle failures.

This understanding allows for the formal definition of the project's Key Performance Indicators (KPIs), which are the metrics used to evaluate the controller's effectiveness :

- Average Waiting Time: The average time, in seconds, that a vehicle spends stopped in a queue before proceeding through the intersection. This is a primary measure of driver frustration and delay.

- Average Queue Length: The average number of vehicles lined up in an approach lane at any given time. This metric is a direct indicator of physical congestion and is critical for preventing "spillback," where a queue extends to block an upstream intersection.

- Throughput: The total number of vehicles that successfully traverse the intersection over the simulation period. This is a measure of the intersection's overall efficiency.

Chapter 2: The Spectrum of Control: From Fixed-Time to Adaptive Systems

Traffic signal controllers have evolved significantly, with each generation reflecting advances in technology and a deeper understanding of traffic dynamics. The journey from simple clockwork mechanisms to AI-driven systems provides the context for this project, highlighting the limitations of traditional methods and the potential of intelligent control.

2.1 The Clockwork Controller: Fixed-Time (Pre-timed) Signals

The most basic form of traffic control is the fixed-time, or pre-timed, signal. These controllers operate on a simple, repeating schedule of green, yellow, and red signal intervals. The duration of each interval and the total cycle length are predetermined based on historical traffic data and do not change in response to real-time conditions.

- Advantages: The primary benefits of fixed-time controllers are their simplicity and low cost. They do not require vehicle detection hardware, making them immune to detector failures and cheaper to install and maintain. Their predictable timing makes them well-suited for coordinating signals in a dense grid, such as a downtown area, where maintaining smooth "green waves" is a priority and traffic patterns are highly regular.

- Disadvantages: The rigidity of fixed-time control is its greatest weakness. It cannot adapt to any fluctuations in traffic flow. This leads to significant inefficiencies, such as forcing vehicles to wait at a red light even when there is no conflicting traffic on the cross-street, particularly during off-peak hours. Furthermore, they are incapable of responding to non-recurring congestion caused by accidents, special events, or road construction, which can exacerbate delays and increase fuel consumption and emissions.

2.2 The Responsive Controller: Vehicle-Actuated Signals

Vehicle-actuated controllers represent a significant step forward by incorporating real-time data. These systems use sensors, such as inductive loops embedded in the pavement or overhead video detectors, to detect the presence of vehicles and adjust signal timings accordingly. This allows the controller to react to changes in traffic demand.

There are two main types of actuated control :

- Semi-Actuated Control: In this configuration, detectors are placed only on the minor approaches of an intersection. The controller defaults to giving a green light to the major road (the "non-actuated" phase) and will only service the minor road when a vehicle is detected there. This design is efficient for intersections of a high-volume arterial street with a low-volume side street, as it minimizes interruptions to the major flow of traffic.

- Fully-Actuated Control: This setup features detectors on all approaches to the intersection. The controller can respond to demand from any direction, and the green time is allocated on a cycle-by-cycle basis. Phases for which no vehicles are detected can be skipped entirely, reallocating that time to other, busier approaches. This makes fully-actuated control highly efficient for isolated intersections where traffic patterns are variable and unpredictable.

While actuated control is far more efficient than fixed-time control, it is not without drawbacks. The systems are more expensive due to the need for detectors, and their performance is contingent on the reliability of these sensors, which can be affected by weather, power issues, and require regular maintenance. They also require more specialized expertise for proper calibration.

2.3 The Intelligent Frontier: Adaptive and AI-based Control

The most advanced systems are known as Adaptive Traffic Control Systems (ATCS). These systems go beyond simple actuation by using advanced algorithms to analyze real-time data and predict short-term traffic patterns. They can optimize signal timings, cycle lengths, and phase sequences dynamically, often coordinating a whole network of intersections to achieve system-wide objectives like minimizing travel time or reducing emissions.

The controllers to be developed in this project—both heuristic-based and reinforcement learning-based—fall into this category. They represent the "intelligence" or the "brain" that can process the data from a standard actuated signal setup and make decisions that are not just reactive, but truly optimal. The historical progression from fixed-time to actuated and now to adaptive control is a direct result of the parallel evolution of technology. Fixed-time controllers were a product of an era of mechanical timers where sensing was impractical. The development of reliable detectors enabled actuated control, allowing systems to react based on simple rules. Today, the availability of inexpensive, powerful microprocessors and advanced sensing technologies (like cameras and AI) makes it possible to implement computationally intensive algorithms like reinforcement learning, allowing the controller to move beyond simple reaction to sophisticated, learned optimization. This project, therefore, represents a practical application of modern computing to solve a classic civil engineering challenge.

To summarize the landscape of control strategies, the following table provides a clear comparison.

| Control Type | Description | Data Requirement | Pros | Cons | Ideal Use Case |
| --- | --- | --- | --- | --- | --- |
| Fixed-Time | Operates on a predetermined, repeating signal cycle. | Historical traffic counts for initial timing plan. | Low cost, simple maintenance, predictable, immune to detector failure. | Inefficient with fluctuating traffic, causes unnecessary delays, cannot adapt to incidents. | Coordinated downtown grids with highly predictable traffic patterns. |
| Semi-Actuated | Major road stays green by default; minor road is serviced only when vehicles are detected. | Detectors on minor road approaches only. | Reduces delay for major road traffic, more efficient than fixed-time. | Can cause long delays for major road if minor road demand is continuous; requires detector maintenance. | Intersection of a major arterial with a low-volume side street. |
| Fully-Actuated | Detectors on all approaches; green time allocated based on real-time demand for all movements. | Detectors on all approaches. | Highly responsive to demand, reduces overall delay, can skip unused phases. | Higher installation and maintenance costs, complex to configure, vulnerable to detector errors. | Isolated intersections with variable and unpredictable traffic patterns. |
| Heuristic / RL Adaptive | Uses advanced algorithms (rules or machine learning) to process real-time data and optimize signal timing for a specific goal (e.g., minimizing delay). | Detectors on all approaches; requires a controller with sufficient processing power. | Potentially optimal performance, can adapt to complex patterns and incidents, continuously learns/improves (RL). | High complexity, can be a "black box" (RL), requires significant data for training (RL), performance depends heavily on algorithm design. | Complex, critical intersections or networks where maximizing efficiency is paramount. |

Part II: The Digital Proving Ground: A Practical Guide to the SUMO Ecosystem

Before an intelligent controller can be deployed in the real world, it must be rigorously tested in a simulated environment. This section provides a comprehensive guide to setting up and using SUMO (Simulation of Urban MObility), the open-source platform that will serve as the digital laboratory for this project.

Chapter 3: Introduction to SUMO (Simulation of Urban MObility)

3.1 What is SUMO?

SUMO is an open-source, highly portable, microscopic, multi-modal traffic simulation package. The term "microscopic" is key: it means that SUMO models the movement and behavior of each individual vehicle, pedestrian, and public transport unit in the network. This level of detail is essential for our project because the performance of a traffic signal controller is determined by its influence on the behavior of these individual agents—their acceleration, deceleration, lane-changing, and waiting times. SUMO provides the realistic, dynamic environment needed to train and validate an intelligent controller.

3.2 Installation and Setup

SUMO is compatible with Windows, macOS, and Linux. The official SUMO documentation provides detailed installation packages and instructions.

- Download and Install SUMO: Visit the official SUMO website (sumo.dlr.de) and download the appropriate version for your operating system. Follow the installer's instructions.

- Set the SUMO_HOME Environment Variable: This is a critical step that many beginners miss. The SUMO_HOME variable must point to the root directory of your SUMO installation (e.g., /usr/share/sumo on Linux or C:\Program Files (x86)\Eclipse\Sumo on Windows). This allows other programs and scripts to locate the SUMO tools.

  - On Windows: Search for "Edit the system environment variables," click "Environment Variables," and add a new System variable named SUMO_HOME with the path to your installation directory.

  - On Linux/macOS: Add the following line to your shell profile file (e.g., ~/.bashrc, ~/.zshrc): export SUMO_HOME="/path/to/your/sumo/installation"

- Verify Installation: Open a new terminal or command prompt and navigate to the bin directory within your SUMO installation. You should see several key executables:

  - sumo: The command-line version of the simulator.

  - sumo-gui: The graphical version, which allows you to visualize the simulation.

  - netedit: A graphical network editor for creating and modifying road networks.

  - netconvert: A tool for converting various network formats into the SUMO .net.xml format.

Chapter 4: Building Your First Intersection

This chapter provides a "Hello, World!" tutorial for SUMO, guiding you through the creation of a simple, four-way signalized intersection from scratch. This process demonstrates the fundamental components of any SUMO scenario. The modular file structure of SUMO is a deliberate and powerful design choice. It separates the static infrastructure (the road network) from the dynamic elements (traffic demand and control logic). This decoupling is what enables scientific, repeatable experiments. One can test different control strategies (by changing the control logic) on the exact same network under identical traffic conditions, ensuring a fair and controlled comparison—a cornerstone of this project's evaluation phase.

4.1 Defining the Arena: Nodes and Edges

The physical layout of a road network is defined by nodes (junctions) and edges (roads). The easiest way to create this is with netedit.

- Launch netedit from the bin directory.

- In "Create Edges" mode, click on the canvas to create a starting node, then click another location to create an ending node. An edge (road) will automatically be created between them.

- Create a four-way intersection by creating two crossing edges. netedit will automatically create a central node where they intersect.

- In "Inspect" mode, you can click on any edge or node to modify its attributes in the left-hand panel, such as the number of lanes, speed limits, or priority.

This visual process generates two underlying XML files :

- Node File (cross.nod.xml): Defines the junctions, each with a unique ID and x,y coordinates.

- Edge File (cross.edg.xml): Defines the roads, specifying their ID, start node (from), end node (to), and properties like numLanes.

These files are then compiled into a single network file (cross.net.xml) using the netconvert tool. netedit does this automatically when you save the network.

4.2 Populating the World: Routes and Traffic Flow

With the network built, the next step is to define the vehicles that will travel on it. This is done in a Route File (.rou.xml). This file contains three main components:

- Vehicle Types (<vType>): Defines the physical characteristics of vehicles, such as length, acceleration, and maximum speed. You can define different types like cars, trucks, or buses.

- Routes (<route>): Defines a specific path through the network as a sequence of edge IDs.

- Vehicles/Flows (<vehicle> or <flow>): Defines the actual traffic. You can add individual vehicles with specific departure times, or define a <flow> which automatically generates a number of vehicles over a time period, following a specific route and vehicle type.

For this project, a simple demand can be created with a higher flow on the east-west road and a lower flow on the north-south road to simulate a main road and a side street.

4.3 Bringing it all Together: The SUMO Configuration File

The SUMO Configuration File (.sumocfg) is the main file used to run a simulation. It's a simple XML file that tells SUMO which network and route files to load. A minimal configuration file looks like this:

XML

<configuration>

<input>

<net-file value="cross.net.xml"/>

<route-files value="cross.rou.xml"/>

</input>

<time>

<begin value="0"/>

</time>

</configuration>

4.4 Adding the Lights

Finally, the traffic light logic needs to be added. This can be done in netedit (by selecting the central junction, right-clicking, and choosing "Create Traffic Light") or by creating an Additional File (.add.xml). This file defines the traffic light programs (

<tlLogic>). Each program consists of a series of phases (<phase>), each with a duration and a state string that defines the signal for each connection in the intersection (e.g., 'G' for green, 'r' for red, 'y' for yellow).

This additional file is then loaded by the .sumocfg file:

XML

<configuration>

<input>

<net-file value="cross.net.xml"/>

<route-files value="cross.rou.xml"/>

<additional-files value="cross.add.xml"/>

</input>

...

</configuration>

With these four files (.net.xml, .rou.xml, .add.xml, .sumocfg), you have a complete, runnable SUMO scenario.

Chapter 5: The TraCI API: Your Key to Dynamic Control

While SUMO can run simulations based on predefined files, the true power for this project comes from dynamic, real-time control. The Traffic Control Interface (TraCI) is the API that makes this possible.

5.1 What is TraCI?

TraCI is a client/server API that allows an external application (in our case, a Python script) to communicate with a running SUMO simulation via a TCP connection. The Python script acts as the client, sending commands and requesting data, while SUMO acts as the server, executing the commands and providing the requested information. This allows the Python script to function as the "brain" of the traffic signal, making decisions at each step of the simulation.

5.2 Setting up the Python Environment

To use TraCI with Python, the official traci library must be installed. This is easily done using pip: pip install traci

A basic Python script to interact with SUMO follows a standard structure :

Python

import traci

import sys

import os

# Ensure SUMO_HOME is in the system path

if 'SUMO_HOME' in os.environ:

tools = os.path.join(os.environ, 'tools')

sys.path.append(tools)

else:

sys.exit("please declare environment variable 'SUMO_HOME'")

# Define the sumo command for launching the simulation

sumo_binary = "sumo-gui" # or "sumo" for command-line

sumo_cmd = [sumo_binary, "-c", "path/to/your/cross.sumocfg"]

# Start the simulation and connect TraCI

traci.start(sumo_cmd)

step = 0

while traci.simulation.getMinExpectedNumber() > 0:

traci.simulationStep()

# --- YOUR CONTROLLER LOGIC GOES HERE ---

step += 1

# Close the TraCI connection

traci.close()

It is critical to call traci.close() at the end of the script; failing to do so can leave the SUMO process running and cause connection errors on subsequent runs.

5.3 Reading the Environment: Getting Data from SUMO

To make intelligent decisions, the controller needs data. TraCI provides a rich set of functions to query the state of the simulation. First, detectors must be placed in the network using netedit. For this project, Induction Loop Detectors (E1) placed near the stop line of each incoming lane are ideal. Once placed, their data can be accessed via TraCI.

5.4 Changing the World: Sending Commands to SUMO

The core of the control logic involves changing the traffic light state. TraCI provides functions specifically for this purpose. The following table distills the vast TraCI documentation into the essential functions needed for this project.

| Function Name | Purpose | Example Usage | Data Returned |
| --- | --- | --- | --- |
| traci.simulationStep() | Advances the simulation by one step. | traci.simulationStep() | None |
| traci.lane.getLastStepHaltingNumber(laneID) | Gets the number of vehicles with speed < 0.1 m/s on a lane. This is our primary measure of queue length. | queue_N = traci.lane.getLastStepHaltingNumber("E2_0") | Integer (number of stopped vehicles) |
| traci.lane.getLastStepMeanSpeed(laneID) | Gets the average speed of vehicles on a lane. | speed_N = traci.lane.getLastStepMeanSpeed("E2_0") | Float (speed in m/s) |
| traci.inductionloop.getLastStepVehicleNumber(detID) | Gets the number of vehicles that passed over a detector in the last step. Useful for measuring flow. | flow = traci.inductionloop.getLastStepVehicleNumber("det_N_0") | Integer |
| traci.trafficlight.getPhase(tlsID) | Gets the index of the current phase in the traffic light's program. | current_phase = traci.trafficlight.getPhase("center") | Integer (phase index) |
| traci.trafficlight.setPhase(tlsID, phaseIndex) | Sets the traffic light to a specific phase index defined in its program. | traci.trafficlight.setPhase("center", 2) | None |
| traci.trafficlight.setRedYellowGreenState(tlsID, stateString) | Sets the traffic light to a custom state string, overriding the predefined program. | traci.trafficlight.setRedYellowGreenState("center", "GrGrGrGr") | None |
| traci.close() | Closes the connection to SUMO and ends the simulation. | traci.close() | None |

Part III: Approach 1 - Intelligent Control with Heuristic Rules

This part details the design and implementation of the first control strategy: a heuristic-based controller. This approach uses simple, human-defined rules to make decisions, offering a significant improvement over fixed-time systems without the complexity of machine learning.

Chapter 6: The Philosophy of Heuristics

6.1 What is a Heuristic?

A heuristic is a practical problem-solving technique that is not guaranteed to be optimal or perfect, but is sufficient for reaching an immediate, short-term goal. It can be thought of as a "rule of thumb," an educated guess, or common sense applied to a problem. In computer science, heuristics are often used when finding an optimal solution is impossible or impractical, favoring speed and simplicity over guaranteed optimality.

6.2 Why Use Heuristics for Traffic Control?

Applying heuristics to traffic signal control offers several distinct advantages, especially when compared to more complex methods like reinforcement learning :

- Simplicity and Speed: Heuristic rules are often intuitive and can be implemented quickly in code. This allows for rapid prototyping and deployment.

- Low Resource Requirement: They do not require large datasets for training or significant computational power for execution, making them a cost-effective solution.

- High Interpretability: The logic of a heuristic controller is transparent. If the system behaves unexpectedly, a human engineer can easily trace the decision-making process and debug the rules. This "white-box" nature is a major advantage for critical infrastructure.

However, heuristics also have inherent limitations :

- Inaccuracy: A simple rule may not be the most accurate or optimal solution in all traffic scenarios, especially complex or unusual ones.

- Bias and Assumptions: The effectiveness of a heuristic is entirely dependent on the assumptions made by its human designer. If these assumptions are flawed, the controller's performance will be suboptimal.

- Scalability: While effective for a single intersection, simple heuristics can struggle to scale to a large, coordinated network of signals where local decisions have far-reaching consequences.

Despite these limitations, a well-designed heuristic can provide a substantial performance boost over basic control systems and serves as an excellent baseline for more advanced methods.

Chapter 7: Implementing a 'Longest Queue First' (LQF) Controller

This chapter provides a detailed walkthrough for building a controller based on one of the most intuitive traffic heuristics: Longest Queue First (LQF).

7.1 The LQF Logic

The core principle of LQF is simple: give the right-of-way (the green light) to the direction of traffic that has the longest queue of waiting vehicles. The goal is to alleviate the worst congestion point at the intersection at any given moment. This is a greedy approach that aims to minimize the maximum delay experienced by any single driver. It is a simplified version of more formal methods like "Max-Pressure" control, which balances the difference in vehicle counts between incoming and outgoing lanes. For this project, the straightforward LQF logic provides a clear and effective demonstration of the heuristic approach.

7.2 System Design

Before writing the code, the system's logic must be mapped out. For a standard four-way intersection, this involves defining the phases and the lanes they control.

- Phase Definition: A typical four-phase cycle for a simple intersection would be:

  - Phase 0: North-South Green (East-West Red)

  - Phase 1: North-South Yellow (East-West Red)

  - Phase 2: East-West Green (North-South Red)

  - Phase 3: East-West Yellow (North-South Red)

- Lane-to-Phase Mapping: The Python script needs to know which lanes correspond to which green phase. For example:

  - Lanes from North and South approaches are served by Phase 0.

  - Lanes from East and West approaches are served by Phase 2.

  - The IDs for these lanes must be obtained from the SUMO network file (e.g., in netedit).

A critical aspect of the design is incorporating constraints. A pure LQF controller that can switch phases at any moment would be highly inefficient. Imagine a scenario where the North-South queue is 10 vehicles and the East-West queue is 9. The controller gives green to N-S. After one second, one N-S car passes and two new cars arrive at the E-W approach. The queues are now 9 and 11, respectively. A pure LQF controller would immediately trigger a phase change. This rapid switching, or "chattering," wastes a significant amount of time in the yellow and all-red clearance intervals, drastically reducing the intersection's throughput. To prevent this, two constraints are essential :

- Minimum Green Time: Once a phase is set to green, it must remain green for a minimum duration (e.g., 10 seconds) to ensure a meaningful number of cars can pass.

- Maximum Green Time: A phase cannot remain green indefinitely, even if its queue is always the longest. A maximum limit (e.g., 60 seconds) ensures that other approaches are not "starved" of service.

7.3 The Python Implementation (Code Walkthrough)

The following provides the structure for a complete, well-commented Python script for the LQF controller.

Python

import traci

import sys

import os

# --- (SUMO setup and traci.start() as in Chapter 5) ---

# Controller Parameters

MIN_GREEN_TIME = 10  # seconds

MAX_GREEN_TIME = 60  # seconds

YELLOW_TIME = 4      # seconds

TLS_ID = "center"    # Traffic Light System ID from SUMO

# Phase definitions (indices from the.add.xml file)

# Example: Phase 0 = NS Green, Phase 2 = EW Green

NS_GREEN_PHASE = 0

NS_YELLOW_PHASE = 1

EW_GREEN_PHASE = 2

EW_YELLOW_PHASE = 3

# Lane IDs for each approach (get these from netedit)

NS_LANES = # Example lane IDs

EW_LANES = # Example lane IDs

# Main simulation loop

green_time_counter = 0

current_phase = traci.trafficlight.getPhase(TLS_ID)

while traci.simulation.getMinExpectedNumber() > 0:

traci.simulationStep()

green_time_counter += 1

# Only make a decision if the minimum green time has passed

if green_time_counter > MIN_GREEN_TIME:

# Get queue lengths for each direction

ns_queue = sum(traci.lane.getLastStepHaltingNumber(lane) for lane in NS_LANES)

ew_queue = sum(traci.lane.getLastStepHaltingNumber(lane) for lane in EW_LANES)

# Decision logic

# Case 1: Current phase is North-South Green

if current_phase == NS_GREEN_PHASE:

# If EW queue is longer OR max green time is reached, switch

if ew_queue > ns_queue or green_time_counter > MAX_GREEN_TIME:

traci.trafficlight.setPhase(TLS_ID, NS_YELLOW_PHASE)

current_phase = NS_YELLOW_PHASE

green_time_counter = -YELLOW_TIME # Reset counter for yellow phase

# Case 2: Current phase is East-West Green

elif current_phase == EW_GREEN_PHASE:

# If NS queue is longer OR max green time is reached, switch

if ns_queue > ew_queue or green_time_counter > MAX_GREEN_TIME:

traci.trafficlight.setPhase(TLS_ID, EW_YELLOW_PHASE)

current_phase = EW_YELLOW_PHASE

green_time_counter = -YELLOW_TIME # Reset counter for yellow phase

# Logic to switch to green after yellow phase

if green_time_counter == 0:

if current_phase == NS_YELLOW_PHASE:

traci.trafficlight.setPhase(TLS_ID, EW_GREEN_PHASE)

current_phase = EW_GREEN_PHASE

elif current_phase == EW_YELLOW_PHASE:

traci.trafficlight.setPhase(TLS_ID, NS_GREEN_PHASE)

current_phase = NS_GREEN_PHASE

traci.close()

This script implements the complete LQF logic with the necessary constraints, providing a robust and effective heuristic-based controller.

Part IV: Approach 2 - Control through Deep Reinforcement Learning

This part transitions to the more advanced and powerful approach: creating a controller that learns its own optimal strategy through trial and error. This involves applying Deep Reinforcement Learning (DRL), a cutting-edge field of artificial intelligence.

Chapter 8: Principles of Reinforcement Learning for Traffic Control

8.1 The RL Framework: An Overview

Reinforcement Learning (RL) is a machine learning paradigm where an intelligent agent learns to make a sequence of optimal actions within an environment in order to maximize a cumulative reward signal. Unlike supervised learning, the agent is not told which action to take. Instead, it learns through a process of trial and error, discovering which actions yield the most reward over the long term. This framework is perfectly suited for traffic control, where an agent can learn to manage signals by observing traffic and receiving rewards based on how its actions affect congestion.

8.2 Defining the Components for Traffic Signal Control

The success of any RL project hinges on the careful definition of its core components. A poorly designed state, action, or reward can make it impossible for the agent to learn an effective policy.

- Environment: The environment is the world in which the agent operates. For this project, the SUMO simulation is the environment. It encompasses the road network, the microscopic behavior of individual vehicles, traffic detectors, and the physics of traffic flow.

- Agent: The agent is the learner and decision-maker. In this context, the agent is the Python script that implements the RL algorithm. It observes the state of the environment and chooses an action, effectively acting as the "brain" of the traffic signal controller.

- Action Space (A): This is the set of all possible actions the agent can take at a given time step. The design of the action space is a critical choice. For a four-way intersection, a simple and effective discrete action space would be :

  - Action 0: Set the next phase to be North-South Green.

  - Action 1: Set the next phase to be East-West Green. After the agent chooses an action, a fixed yellow time is automatically applied before the new green phase begins. This simplifies the agent's task, as it doesn't need to learn to control the yellow phase.

- State Space (S): The state is the agent's perception of the environment. The agent uses the state to decide which action to take. The state representation must contain enough information for the agent to make an intelligent decision, but not so much that the learning problem becomes intractable. Common representations include :

  - Queue Length Vector: A simple vector containing the number of waiting vehicles in each approach lane (e.g., ``). This is simple but may not capture the full dynamics of traffic.

  - Discretized Position/Speed Matrix: A more complex and powerful representation. Each incoming lane is divided into a number of cells of a certain length (e.g., 5 meters). The state is then a matrix or flattened vector where each element represents a cell and its value indicates the presence (1 or 0) or speed of a vehicle in that cell. This gives the agent a spatial understanding of the traffic situation and is well-suited for processing by a neural network. This project will use this more advanced representation.

- Reward Function (R): The reward function provides the feedback signal to the agent. It is a numerical value that indicates how good the agent's last action was in terms of achieving the project's goals. The agent's objective is to learn a policy that maximizes the cumulative reward over time. The design of the reward function is perhaps the most critical element, as it directly shapes the agent's behavior. Common reward functions in traffic signal control include :

  - Change in Cumulative Waiting Time: reward = total_wait_time_at_previous_step - total_wait_time_at_current_step. This directly rewards actions that reduce the total time vehicles spend waiting. It is a very effective and direct measure of performance.

  - Negative of Queue Length: reward = -sum(all_queue_lengths). This penalizes the agent for allowing long queues to form.

  - Pressure: reward = -(number_of_vehicles_on_incoming_lanes - number_of_vehicles_on_outgoing_lanes). This encourages the agent to balance the traffic flow across the intersection.

For this project, the change in cumulative waiting time will be used as the reward function, as it provides a direct and intuitive signal for minimizing delay. The following table summarizes these crucial design choices.

| RL Component | Definition/Example | Pros | Cons |
| --- | --- | --- | --- |
| State | Discretized lane occupancy matrix: Each incoming lane is divided into cells. The state is a vector where each element is 1 if a car is in the corresponding cell, 0 otherwise. | Provides rich, spatial information about vehicle positions. Well-suited for deep learning models (CNNs/DNNs). Captures more than just the queue at the stop line. | High-dimensional state space. Requires more complex neural network architecture. Can be computationally more intensive. |
| Action | Discrete choice of the next green phase: Action 0 = Set NS-Green, Action 1 = Set EW-Green. Yellow phase handled automatically. | Simple, discrete action space is easier for the agent to learn. Avoids the complexity of learning variable green times. | Less flexible than learning green time duration. The fixed phase duration is a hyperparameter that must be tuned. |
| Reward | Change in cumulative waiting time: reward = wait_time(t-1) - wait_time(t). | Directly incentivizes the agent to reduce vehicle waiting time, which is the primary project goal. Provides a dense reward signal at every step. | Can be noisy. A single car stopping or starting can cause fluctuations. Requires tracking the waiting time of all vehicles. |

Chapter 9: From Tables to Networks: Q-Learning and the Rise of DQN

9.1 The Intuition: Tabular Q-Learning

The foundational algorithm in value-based reinforcement learning is Q-learning. Its goal is to learn an optimal

action-value function, denoted as Q(s,a). This function predicts the total expected future discounted reward an agent can receive by taking action a in state s and then following the optimal policy thereafter.

In its simplest form, Q-learning uses a Q-table, which is a large lookup table with a row for every possible state and a column for every possible action. The cell at (s, a) stores the learned Q-value. The agent updates this table using the

Bellman equation, which iteratively refines the Q-value estimates based on experience :

Q(st​,at​)←Q(st​,at​)+α[rt+1​+γamax​Q(st+1​,a)−Q(st​,at​)]

Here:

- st​,at​ are the state and action at the current time step t.

- rt+1​,st+1​ are the reward and next state received after taking the action.

- α is the learning rate, which controls how much the new information overrides the old Q-value.

- γ is the discount factor, which determines the importance of future rewards versus immediate rewards.

However, this tabular approach suffers from the curse of dimensionality. For a problem like traffic control, the state space (our discretized matrix of vehicle positions) is enormous. The number of possible states is astronomical, making it impossible to create, store, or populate a Q-table. The agent would never visit most states even once, so it could never learn their values.

9.2 The Solution: Deep Q-Networks (DQN)

To overcome the limitations of tabular Q-learning, Deep Q-Networks (DQN) were introduced. The core innovation of DQN is to use a deep neural network as a function approximator to estimate the Q-function, completely replacing the Q-table. The neural network takes the high-dimensional state (e.g., our vehicle position matrix) as its input and outputs a vector of Q-values, one for each possible action.

Simply replacing the table with a neural network introduces severe instabilities during training. DQN implements two key techniques to solve this :

- Experience Replay: The agent's experiences—tuples of (state, action, reward, next_state, done)—are stored in a large buffer called a replay memory. Instead of training the network on consecutive, highly correlated experiences, the algorithm samples random mini-batches of experiences from this memory to train the network. This breaks the temporal correlations in the data, leading to much more stable and effective learning.

- Target Network: DQN uses two separate neural networks. The main network is the one whose weights are constantly being updated through training. The target network is a clone of the main network whose weights are frozen. The target network is used to calculate the target Q-value in the Bellman equation (max Q(s', a')). Its weights are only updated to match the main network's weights periodically (e.g., every few hundred steps). This provides a stable target for the main network to learn towards, preventing the optimization from chasing a rapidly changing target and diverging.

Chapter 10: Building and Training a DQN Agent

This chapter provides a practical guide to implementing the DQN agent in Python and training it to control the SUMO intersection.

10.1 Setting up the Python Environment

This project requires a deep learning framework. Both TensorFlow (with the Keras API) and PyTorch are excellent choices. Installation is typically done via pip: pip install tensorflow or pip install torch

10.2 The DQN Architecture

A simple but effective neural network for this task is a fully connected Deep Neural Network (DNN), also known as a Multi-Layer Perceptron (MLP).

- Input Layer: The number of neurons in the input layer must match the size of the state representation. If the state is a flattened matrix of 80 cells representing vehicle presence, the input layer will have 80 neurons.

- Hidden Layers: Two to three hidden layers are typically sufficient. A common architecture uses a few hundred neurons per layer (e.g., 256 or 400) with the Rectified Linear Unit (ReLU) activation function, which helps with training deep networks.

- Output Layer: The output layer has a number of neurons equal to the number of possible actions (e.g., 2 for our "NS Green" vs. "EW Green" choice). It uses a linear activation function, as it needs to output the raw, unbounded Q-values for each action.

10.3 The Agent Class (Code Walkthrough)

A well-structured DQNAgent class is essential for managing the complexity of the algorithm. The following is a conceptual outline of the class in Python.

Python

import numpy as np

import random

from collections import deque

# import tensorflow as tf # or import torch

class DQNAgent:

def __init__(self, state_size, action_size):

self.state_size = state_size

self.action_size = action_size

self.replay_memory = deque(maxlen=2000) # Experience replay buffer

# Hyperparameters

self.gamma = 0.95    # Discount factor

self.epsilon = 1.0   # Exploration rate

self.epsilon_min = 0.01

self.epsilon_decay = 0.995

self.learning_rate = 0.001

# Build the main and target neural networks

self.main_network = self._build_network()

self.target_network = self._build_network()

self.update_target_network()

def _build_network(self):

# Code to build a Keras/TensorFlow or PyTorch model

# Input layer: self.state_size

# Hidden layers: e.g., Dense(256, activation='relu')

# Output layer: Dense(self.action_size, activation='linear')

# Compile with an optimizer like Adam and a loss function like Mean Squared Error

pass

def update_target_network(self):

# Copies weights from main_network to target_network

self.target_network.set_weights(self.main_network.get_weights())

def remember(self, state, action, reward, next_state, done):

# Store experience in replay memory

self.replay_memory.append((state, action, reward, next_state, done))

def act(self, state):

# Epsilon-greedy action selection

if np.random.rand() <= self.epsilon:

return random.randrange(self.action_size) # Explore

q_values = self.main_network.predict(state)

return np.argmax(q_values) # Exploit

def replay(self, batch_size):

# Train the network using experience replay

if len(self.replay_memory) < batch_size:

return

minibatch = random.sample(self.replay_memory, batch_size)

for state, action, reward, next_state, done in minibatch:

target = reward

if not done:

# Use target network to predict future reward

target = (reward + self.gamma *

np.amax(self.target_network.predict(next_state)))

# Get current Q-values from main network

target_f = self.main_network.predict(state)

# Update the Q-value for the action that was taken

target_f[action] = target

# Train the main network

self.main_network.fit(state, target_f, epochs=1, verbose=0)

# Decay epsilon

if self.epsilon > self.epsilon_min:

self.epsilon *= self.epsilon_decay

10.4 The Main Training Loop

The main script orchestrates the entire training process by looping through episodes and time steps, connecting the agent to the SUMO environment.

- Initialization: Start SUMO with TraCI, create an instance of the DQNAgent.

- Episode Loop: Loop for a specified number of training episodes (e.g., 1000).

- Reset: At the start of each episode, reset the SUMO simulation to get a fresh start and get the initial state.

- Time Step Loop: Within each episode, loop for a maximum number of steps. a. Get State: Use TraCI to gather the information needed for the state representation. b. Choose Action: Call the agent's act() method to get the next action based on the current state. c. Perform Action: Use TraCI to set the new traffic light phase in SUMO. Let the simulation run for a fixed duration (e.g., 10 seconds for the green phase plus 4 for yellow). d. Observe Outcome: After the action is complete, get the reward (change in wait time) and the next_state from SUMO. e. Store Experience: Call the agent's remember() method to store the (state, action, reward, next_state) tuple. f. Train: Call the agent's replay() method to train the main network on a batch of experiences.

- Update Target Network: Periodically (e.g., every 5 episodes), call update_target_network() to sync the target network with the main network.

- Save Model: Periodically save the weights of the trained main network.

This training process embodies the crucial balance between exploration and exploitation. Initially, with a high epsilon, the agent's actions are mostly random. This allows it to explore the environment and populate its replay memory with a diverse set of experiences. As training progresses, epsilon decays, and the agent increasingly exploits its learned knowledge by choosing actions that its Q-network predicts will yield the highest reward. This structured transition from broad exploration to focused exploitation is fundamental to the agent's ability to converge on a robust and effective control policy.

Part V: Evaluation and Comparative Analysis

After developing the controllers, the final step is to rigorously evaluate their performance. This part outlines a systematic approach to measure the effectiveness of each controller and compare them against a baseline, providing quantitative evidence of their success.

Chapter 11: Measuring What Matters

To perform a fair comparison, a consistent method for data collection and analysis is required. SUMO provides powerful tools for logging detailed performance metrics during a simulation run.

11.1 Configuring SUMO for Data Collection

By adding specific command-line options or XML configuration tags, SUMO can be instructed to generate detailed output files. The most relevant for this project are:

- Queue Output (--queue-output <FILE>): This option generates an XML file containing detailed information about queues at each time step. For each lane, it logs the queueing_time (total waiting time of vehicles in the queue) and queueing_length (the physical length of the queue in meters). This is the primary source for calculating average wait time and queue length.

- Trip Information Output (--tripinfo-output <FILE>): This file logs a summary for every vehicle that completes its journey during the simulation. It includes metrics like traveltime, timeLoss (waiting time), and departDelay. This provides an alternative, vehicle-centric way to measure performance.

- Summary Output (--summary-output <FILE>): Provides aggregated statistics for the entire simulation run, including the total number of vehicles that completed their trips, which is a direct measure of throughput.

11.2 Post-Processing with Python

The raw XML outputs from SUMO must be processed to calculate the final aggregate KPIs. Python, with libraries like pandas for data manipulation and xml.etree.ElementTree for parsing, is ideal for this task. A post-processing script would typically:

- Parse the queue-output.xml file.

- For each time step, sum the queueing_time and queueing_length across all relevant lanes.

- Average these sums over the entire duration of the simulation to get the Average Waiting Time and Average Queue Length.

- Parse the summary-output.xml file to extract the total number of ended trips to calculate Throughput (in vehicles per hour).

These calculations provide the core metrics needed to evaluate and compare the controllers.

Chapter 12: Head-to-Head: Comparing Controller Performance

12.1 Experimental Design

A robust evaluation requires testing the controllers under various conditions to assess their adaptability. The experiment should consist of three main controller configurations run on the exact same SUMO network:

- Baseline Controller: A standard fixed-time controller with a reasonable, pre-calculated cycle length (e.g., 90 seconds total, with 42 seconds green for each direction and 3 seconds yellow). This represents the "unintelligent" status quo.

- Heuristic Controller: The 'Longest Queue First' (LQF) controller developed in Part III.

- RL Controller: The trained Deep Q-Network (DQN) agent from Part IV. For evaluation, the agent's exploration rate (epsilon) should be set to zero or a very small value to ensure it is consistently exploiting its learned policy.

These three controllers should be tested against at least two different traffic demand scenarios in the .rou.xml file:

- Scenario 1: Balanced Peak Hour: Heavy, but relatively equal, traffic flow on all approaches.

- Scenario 2: Unbalanced Flow: Heavy traffic on the main east-west arterial and light, sporadic traffic on the north-south side street. This tests the controller's ability to adapt to asymmetric conditions.

12.2 Presenting and Interpreting Results

The results of the simulations should be compiled into a clear, comparative format. A summary table is the most effective way to present the final performance data. Bar charts are also excellent for visualizing the differences between the controllers for each KPI.

| Controller | Traffic Scenario | Avg. Wait Time (s/veh) | Avg. Queue Length (vehicles) | Throughput (veh/hr) |
| --- | --- | --- | --- | --- |
| Fixed-Time | Balanced Peak | 45.2 | 12.5 | 1850 |
| LQF Heuristic | Balanced Peak | 28.6 | 7.1 | 2100 |
| DQN Agent | Balanced Peak | 22.1 | 5.8 | 2250 |
| Fixed-Time | Unbalanced Flow | 55.8 | 15.3 (on main road) | 1600 |
| LQF Heuristic | Unbalanced Flow | 25.1 | 6.5 | 2050 |
| DQN Agent | Unbalanced Flow | 15.5 | 4.2 | 2350 |

Export to Sheets

(Note: The values in this table are illustrative examples of expected outcomes.)

Analysis of Results: The final step is to interpret these results.

- In the Balanced Peak scenario, both the LQF and DQN controllers would be expected to significantly outperform the fixed-time baseline. The DQN may show a slight edge over the LQF by learning more subtle dynamics than the simple "longest queue" rule.

- The Unbalanced Flow scenario is where the intelligent controllers truly shine. The fixed-time controller wastes a large portion of its cycle giving green time to an empty side street, causing major delays on the main road. Both the LQF and DQN controllers would quickly adapt, dedicating most of the green time to the main road and only servicing the side street when vehicles are actually present. The DQN might perform even better than the LQF if it learns to anticipate platoons of vehicles arriving on the main road, a pattern the purely reactive LQF controller cannot predict. This analysis, backed by the quantitative data, provides a powerful validation of the project's success.

Part VI: Advanced Concepts and Future Directions

This project provides a solid foundation in building and evaluating intelligent traffic signal controllers. However, the field is vast and rapidly evolving. This final part offers a glimpse into more advanced topics and provides resources for continued exploration.

Chapter 13: Beyond a Single Intersection

13.1 The Challenge of Coordination

Optimizing a single intersection in isolation can sometimes have negative consequences for the surrounding network. For example, an efficient controller might create a large, fast-moving platoon of vehicles (a "green wave") that then arrives at the next downstream intersection all at once, overwhelming it and causing a queue to form. True city-scale traffic optimization requires coordination between adjacent signals.

13.2 Introduction to Multi-Agent Reinforcement Learning (MARL)

Multi-Agent Reinforcement Learning (MARL) is the extension of RL to problems with multiple interacting agents. In the context of traffic control, each intersection in a network can be controlled by its own independent agent. These agents can learn to cooperate and achieve a global objective. Common MARL approaches include:

- Independent Learners: Each agent treats the other agents as part of the environment. This is the simplest approach but can be unstable as the environment is constantly changing from each agent's perspective.

- Shared Information: Agents can communicate their state or intended actions to their neighbors. This allows them to make more informed, coordinated decisions.

- Shared Reward: All agents can be trained to optimize a single, global reward signal (e.g., the average travel time across the entire network). This encourages emergent cooperative behavior.

MARL is a highly active area of research and represents the next frontier in intelligent traffic management.

Chapter 14: Project Resources and Next Steps

This guide provides a comprehensive starting point. To delve deeper, the following resources are invaluable.

- Official Documentation:

  - SUMO Documentation: The definitive source for all SUMO features, tools, and file formats.

  - TraCI Python API Documentation: A detailed reference for all available TraCI commands.

- Key GitHub Repositories: Exploring existing code is one of the best ways to learn. These repositories provide excellent implementations of RL for traffic control with SUMO:

  - SUMO-RL: A widely used library that provides a simple interface to create RL environments with SUMO, compatible with popular libraries like Gymnasium and Stable-Baselines3.

  - Eco-Light: An extension of SUMO-RL that focuses on optimizing for CO2 emissions.

  - RESCO (Reinforcement Learning Benchmarks for Traffic Signal Control): A benchmark suite for evaluating MARL algorithms on realistic, large-scale traffic scenarios.

  - Other notable projects:.

- Next Steps for Your Project:

  - Experiment with the Reward Function: Implement and test a different reward function, such as one based on intersection pressure or CO2 emissions, and analyze how it changes the agent's learned behavior.

  - Try Advanced RL Algorithms: The DQN implemented here is a foundational algorithm. Explore more modern and powerful algorithms like Proximal Policy Optimization (PPO) or Advantage Actor-Critic (A2C), which can offer more stable and efficient training.

  - Tackle a Multi-Intersection Scenario: Expand your SUMO network to include two or three coordinated intersections and attempt to implement a basic MARL strategy.

  - Use Real-World Data: Many cities provide open traffic data. Try to create a SUMO scenario that is calibrated to reflect real-world traffic counts and patterns, making your simulation even more realistic.

By completing this project, you have gained a deep, practical understanding of traffic flow theory, simulation, and the application of both heuristic and state-of-the-art AI control strategies. The skills and knowledge acquired are directly applicable to the ongoing global effort to create smarter, more efficient, and more sustainable urban transportation systems.

Sources used in the report

computerresearch.org

Intelligent Vehicular Traffic Control System using Priority Longest

Opens in a new window

dot.ca.gov

Advanced Traffic Signal Control Algorithms - Caltrans

Opens in a new window

zenodo.org

comparison of fixed-time vs. actuated traffic signals - Zenodo

Opens in a new window

demo.webassign.net

Fundamental Principles of Traffic Flow - WebAssign

Opens in a new window

numberanalytics.com

Mastering Traffic Flow Fundamentals - Number Analytics

Opens in a new window

en.wikipedia.org

Heuristic routing - Wikipedia

Opens in a new window

en.wikibooks.org

Fundamentals of Transportation/Traffic Flow - Wikibooks, open books for an open world

Opens in a new window

scribd.com

2A - Fundamentals of Traffic Flow | PDF | Speed - Scribd

Opens in a new window

en.wikipedia.org

Traffic light control and coordination - Wikipedia

Opens in a new window

courses.washington.edu

Fundamentals of Traffic Flow and Queuing Theory

Opens in a new window

nacto.org

Fixed vs. Actuated Signalization - NACTO

Opens in a new window

medium.com

Why Traditional Methods of Traffic Control and Fixed-Time Actuation Are No Longer Effective for a City like Freetown — By Moses Joseph Lappia, B.Eng (Hons). - Medium

Opens in a new window

greenlighttrafficengineering.com

A Guide to Types of Traffic Signals - Webflow HTML Website Template

Opens in a new window

en.wikipedia.org

Fundamental diagram of traffic flow - Wikipedia

Opens in a new window

ops.fhwa.dot.gov

Traffic Signal Timing Manual: Chapter 5 - Office of Operations

Opens in a new window

ce.memphis.edu

Queuing Theory and Traffic Flow

Opens in a new window

mdpi.com

A Robust Adaptive Traffic Signal Control Algorithm Using Q-Learning under Mixed Traffic Flow - MDPI

Opens in a new window

mdpi.com

Adaptive Deep Q-Network Algorithm with Exponential Reward Mechanism for Traffic Control in Urban Intersection Networks - MDPI

Opens in a new window

github.com

AndreaVidali/Deep-QLearning-Agent-for-Traffic-Signal-Control: A framework where a deep Q-Learning Reinforcement Learning agent tries to choose the correct traffic light phase at an intersection to maximize traffic efficiency. - GitHub

Opens in a new window

fhwa.dot.gov

signalized intersections - Traffic Flow Theory

Opens in a new window

ceur-ws.org

Learning From Your Virtual Neighbors: a Reinforcement Learning Approach to Traffic Signal Control - CEUR-WS.org

Opens in a new window

icicel.org

a study on the traffic signal control using the extended deep q network - ICIC Express Letters

Opens in a new window

traffic-signal-control.github.io

Reinforcement Learning for Traffic Signal Control

Opens in a new window

researchgate.net

(PDF) DQN-BASED TRAFFIC SIGNAL CONTROL SYSTEMS - ResearchGate

Opens in a new window

tandfonline.com

First steps towards real-world traffic signal control optimisation by reinforcement learning

Opens in a new window

cdnsciencepub.com

Adaptive traffic signal control using deep Q-learning: case study on optimal implementations

Opens in a new window

ojs.aaai.org

Guidelines for Action Space Definition in Reinforcement Learning-Based Traffic Signal Control Systems

Opens in a new window

pmc.ncbi.nlm.nih.gov

Deep Reinforcement Learning for Traffic Signal Control Model and Adaptation Study - PMC

Opens in a new window

arxiv.org

Traffic Light Control with Reinforcement Learning - arXiv

Opens in a new window

sumo.dlr.de

SUMO Documentation - Eclipse SUMO - Simulation of Urban MObility

Opens in a new window

sumo.dlr.de

Tutorials - SUMO Documentation - Eclipse SUMO - Simulation of Urban MObility

Opens in a new window

sumo.dlr.de

TraCI - SUMO Documentation

Opens in a new window

reddit.com

SUMO Tutorial : r/SumoSimulation - Reddit

Opens in a new window

pypi.org

traci · PyPI

Opens in a new window

sumo.dlr.de

TraCI4Traffic Lights - Eclipse SUMO - Simulation of Urban MObility

Opens in a new window

cse.iitkgp.ac.in

Tutorial 6: Traffic Light Control Using TraCI - CSE IIT KGP

Opens in a new window

bpb-us-e1.wpmucdn.com

SUMO: an introduction from a beginner

Opens in a new window

eclipse.dev

Eclipse SUMO - Simulation of Urban MObility

Opens in a new window

howtocodetutorial.wordpress.com

Controlled intersection in SUMO - Tutorials - WordPress.com

Opens in a new window

medium.com

Mastering SUMO Traffic Simulation with TraCI | by Taher Almoussali - Medium

Opens in a new window

researchgate.net

(PDF) A stable longest queue first signal scheduling algorithm for an ...

Opens in a new window

journals.plos.org

Deep Q-network-based traffic signal control models | PLOS One

Opens in a new window

sumo.dlr.de

FAQ - SUMO Documentation - Eclipse SUMO - Simulation of Urban MObility

Opens in a new window

sumo.dlr.de

QueueOutput - SUMO Documentation

Opens in a new window

lucasalegre.github.io

SUMO-RL 1.4.5 documentation - Lucas N. Alegre

Opens in a new window

github.com

pagand/Eco-Light: Environment friendly traffic signal control with deep reinforcement learning - GitHub

Opens in a new window

github.com

mschrader15/reinforcement-learning-sumo - GitHub

Opens in a new window

mikecarruego.medium.com

Choosing the Right Algorithm: Machine Learning vs. Heuristics | by Miguel Carruego

Opens in a new window

mdpi.com

Traffic Signal Control via Reinforcement Learning: A Review on Applications and Innovations - MDPI

Opens in a new window

mdpi.com

Intelligent Meta-Heuristic-Based Optimization of Traffic Light Timing Using Artificial Intelligence Techniques - MDPI

Opens in a new window

github.com

TJ1812/Adaptive-Traffic-Signal-Control-Using-Reinforcement-Learning: This is an application exploiting principles of Deep Reinforcement Learning. The Deep Neural Network is trained to approximate the Bellman Equation (Q-Learning). - GitHub

Opens in a new window

github.com

GangSuUGA/StageLight: SUMO Pytorch Deep Reinforcement Learning Traffic Signal Control - GitHub

Opens in a new window

github.com

Pi-Star-Lab/RESCO: Reinforcement Learning Benchmarks for Traffic Signal Control (RESCO) - GitHub

Opens in a new window

sumo.dlr.de

TraCI4Traffic Lights - SUMO Documentation

Opens in a new window

mdpi.com

Improved Deep Reinforcement Learning for Intelligent Traffic Signal ...

Opens in a new window

sumo.dlr.de

Quick Start - SUMO Documentation

Opens in a new window

Sources read but not used in the report

stackoverflow.com

How to implement longest-queue-first rule for traffic lights in SUMO or FLOW?

Opens in a new window

opentransportationjournal.com

Estimating Queue at Traffic Signals - The Open Transportation Journal

Opens in a new window

academic.oup.com

Hyper-heuristic algorithm for traffic flow-based vehicle routing problem with simultaneous delivery and pickup | Journal of Computational Design and Engineering | Oxford Academic

Opens in a new window

researchgate.net

A Heuristic Adaptive Traffic Control Algorithm for Signalized Intersections - ResearchGate

Opens in a new window

pmc.ncbi.nlm.nih.gov

Heuristic Optimal Scheduling for Road Traffic Incident Detection Under Computational Constraints - PMC

Opens in a new window

mdpi.com

Heuristic Fuzzy Approach to Traffic Flow Modelling and Control on ...

Opens in a new window

easts.info

Queue Length Estimation Algorithm for Signalized Intersection Using Sectional Travel Time Information

Opens in a new window

mdpi.com

Double Deep Q-Network with a Dual-Agent for Traffic Signal Control - MDPI

Opens in a new window

cecs.ucf.edu

Real-time Signal Queue Length Prediction Using Long Short-Term Memory Neural Network

Opens in a new window

researchgate.net

(PDF) Traffic Light Control with Reinforcement Learning - ResearchGate

Opens in a new window

pmc.ncbi.nlm.nih.gov

Deep Q-network-based traffic signal control models - PMC

Opens in a new window

onlinepubs.trb.org

Algorithm for Estimating Queue Lengths and Stop Delays at Signalized Intersections - Transportation Research Board

Opens in a new window

udemy.com

Free Tutorial - SUMO Traffic Simulator - Udemy

Opens in a new window

sumo.dlr.de

Protocol - SUMO Documentation

Opens in a new window

github.com

sumo/docs/web/docs/TraCI.md at main - GitHub

Opens in a new window

sumo.dlr.de

neteditUsageExamples - SUMO Documentation

Opens in a new window

youtube.com

SUMO Traffic Simulator Part1.1: Installation & Create a Simple Network - YouTube

Opens in a new window

stackoverflow.com

Create traffic light in SUMO and OMNeT++ - veins - Stack Overflow

Opens in a new window

github.com

CIV8740/manual-sumo.md at master - GitHub

Opens in a new window

eclipse.org

Re: [sumo-user] stoplines placed on the intersections - Eclipse

Opens in a new window

eclipse.org

[sumo-user] Realistic behavior for approaching intersections - Eclipse

Opens in a new window

sumo.dlr.de

Intersections - SUMO Documentation - Eclipse SUMO - Simulation of Urban MObility

Opens in a new window

stackoverflow.com

How to use traffic data from sumo as an RL environment to be used in python?

Opens in a new window

arxiv.org

SUMO-MCP: Leveraging the Model Context Protocol for Autonomous Traffic Simulation and Optimization - arXiv

Opens in a new window

aii.transportation.org

Automated Traffic Signal Performance Measures - AASHTO Innovation Initiative

Opens in a new window

ops.fhwa.dot.gov

Automated Traffic Signal Performance Measures - FHWA Office of Operations

Opens in a new window

diva-portal.org

ROUNDABOUT MICROSIMULATION USING SUMO - DiVA portal

Opens in a new window

github.com

Question about interpretation of rewards/environment evaluation · Issue #60 · LucasAlegre/sumo-rl - GitHub

Opens in a new window

researchgate.net

Travel time calculation in the SUMO simulator. - ResearchGate

Opens in a new window

docs.lib.purdue.edu

Performance Measures for Traffic Signal Systems: An Outcome-Oriented Approach

Opens in a new window

tandfonline.com

Full article: A deep reinforcement learning based hyper-heuristic for modular production control - Taylor & Francis Online

Opens in a new window

arxiv.org

Robustness of Reinforcement Learning-Based Traffic Signal Control under Incidents: A Comparative Study - arXiv

Opens in a new window

ops.fhwa.dot.gov

What are Automated Traffic Signal Performance Measures? - FHWA Office of Operations

Opens in a new window

udottraffic.utah.gov

Automated Traffic Signal Performance Measures

Opens in a new window

kittelson.com

The Current State of Automated Traffic Signal Performance Measures

Opens in a new window

pmc.ncbi.nlm.nih.gov

A review of reinforcement learning based hyper-heuristics - PMC

Opens in a new window

Thoughts